New_Protocol_for_AllinOne

09/22/2019

| RF board Serial Number |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Board | Board type |  | frequency |  | Tx¦ Rx | model | month | year | index |  |
| 980AD | RF (UTF-8) |  | 0980 |  | 1 2 | A (UTF-8) | 09 | 19 | 00 | 01 |
| code | 0x52 | 0x46 | 0x03 | 0xD4 | 0x12 | 0x41 | 0x09 | 0x13 | 0x00 | 0x01 |

| PM board Serial Number |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Board | Board type |  | Cyclone V | # of ADC | month | year | index |  |
| PM504 | PM(UTF-8) |  | 05 | 04 | 09 | 19 | 00 | 01 |
| code | 0x50 | 0x4D | 0x05 | 0x04 | 0x09 | 0x13 | 0x00 | 0x01 |

| Kit Info Examples(index=1, hex code) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 240AD | 580AD2 | 980AD2 | 2400AD2 | 2400T1R4 | 2400T2R4 |
| Header | FA 0D | FA 0D | FA 0D | FA 0D | FA 0D | FA 0D |
| MCU Info | ED 04 | ED 04 | ED 04 | ED 04 | ED 04 | ED 04 |
|  | 50 4D | 50 4D | 50 4D | 50 4D | 50 4D | 50 4D |
|  | 05 04 | 05 04 | 05 04 | 05 04 | 05 04 | 05 04 |
|  | 09 13 | 09 13 | 09 13 | 09 13 | 09 13 | 09 13 |
|  | 00 01 | 00 01 | 00 01 | 00 01 | 00 01 | 00 01 |
| RF Info | F2 05 | F2 05 | F2 05 | F2 05 | F2 05 | F2 05 |
|  | 52 46 | 52 46 | 52 46 | 52 46 | 52 46 | 52 46 |
|  | 00 F0 | 02 40 | 03 D4 | 09 60 | 18 00 | 18 00 |
|  | 11 41 | 12 41 | 12 41 | 12 41 | 14 41 | 24 41 |
|  | 09 13 | 09 13 | 09 13 | 09 13 | 09 13 | 09 13 |
|  | 00 01 | 00 01 | 00 01 | 00 01 | 00 01 | 00 01 |
| Footer | FB BF | FB BF | FB BF | FB BF | FB BF | FB BF |

|  | Device information from kit to PC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Information | 15 | 14 | 13 | 12 | 11 | 10 | 09 | 08 | 07 | 06 | 05 | 04 | 03 | 02 | 01 | 00 |
| 1 | Header | Header ID: 0xFA |  |  |  |  |  |  |  | Packet Length: words(16bit) |  |  |  |  |  |  |  |
| 2 | MCU Board Information (use UTF-8) | MCU ID: 0xED (0x4D(UTF-8 M)+A0) |  |  |  |  |  |  |  | Length: Following words |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | RF Board Information (use UTF-8) | RF ID: 0xF2 (0x52(UTF-8 R)+A0) |  |  |  |  |  |  |  | Length: Following words |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Version | Version ID: 0xF6 (0x56(UTF-8 V)+A0) |  |  |  |  |  |  |  | Firmware Version number |  |  |  |  |  |  |  |
| 13 | Error Info | Error ID: 0xE5(0x45(UTF-8 E)+A0) |  |  |  |  |  |  |  | 1: error, 0:no error (reserved for WIFI ) |  |  |  |  |  |  |  |
| 14 | … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … | … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| last | PacketEnd | PacketEnd ID: 0xFBBF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Settings from Host to Kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | bits | 15 | 14 | 13 | 12 | 11 | 10 | 09 | 08 | 07 | 06 | 05 | 04 | 03 | 02 | 01 | 00 |
| 1 | Info Request | FA |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |
| General settings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Modulation | E1 |  |  |  |  |  |  |  | x | x | x | x |  |  |  |  |
| 2 | SweepTime | E2 |  |  |  |  |  |  |  | x | x | x | x |  |  |  |  |
| 3 | Sampling No | E3 |  |  |  |  |  |  |  | x | x | x | x |  |  |  |  |
| 4 | Tx Channel | E4 |  |  |  |  |  |  |  | x | x | x | x |  |  |  |  |
| 5 | Rx Channel | E5 |  |  |  |  |  |  |  | x | x | x | x |  |  |  |  |
| 6 | Year | E6 (current year-1980) |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |
| 7 | Month | E7 |  |  |  |  |  |  |  | x | x | x | x |  |  |  |  |
| 8 | Day | E8 |  |  |  |  |  |  |  | x | x | x |  |  |  |  |  |
| 9 | hour | E9 |  |  |  |  |  |  |  | x | x | x |  |  |  |  |  |
| 10 | minute | EA |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |
| 11 | second | EB (second/2) |  |  |  |  |  |  |  | x | x | x |  |  |  |  |  |
| PLL settings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Sweepstop_Hi | D1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | sweepstop_Lo | D2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | register03_Hi | C1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | register03_Mi | C2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | register03_Lo | C3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | register04_Hi | C4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | register04_Mi | C5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | register04_Lo | C6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | register0A_Hi | C7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | register0A_Mi | C8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | register0A_Lo | C9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | register0C_Hi | CA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | register0C_Mi | CB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | register0C_Lo | CC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | register0D_Hi | CD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | register0D_Mi | CE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | register0D_Lo | CF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Modulation |  |  |  |  |
| --- | --- | --- | --- | --- |
| code |  |  |  | description |
| 0 | 0 | 0 | 0 | FMCW__sawtooth |
| 0 | 0 | 0 | 1 | FMCW__triangle (reserved) |
| 0 | 0 | 1 | 0 | FMCW__square (FSK)(reserved) |
| 0 | 0 | 1 | 1 | CW |
| 0 | 1 | 0 | 0 | spare |

| Sweep Time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| code |  |  |  | Sweep time | Base Sampling number | Max bandwidth 24GHz | Max bandwidth 5898 |
| 0 | 0 | 0 | 0 | 125us | 256 | 250MHz | 50MHz |
| 0 | 0 | 0 | 1 | 250us | 512 | 250MHz | 100MHz |
| 0 | 0 | 1 | 0 | 500us | 1024 | 1000MHz | 100MHz |
| 0 | 0 | 1 | 1 | 1000us | 2048 | 2500MHz | 400MHz |
| 0 | 1 | 0 | 0 | 2000us | 4096 | 2500MHz | 400MHz |
| 0 | 1 | 0 | 1 | 4000us | 8192 | 2500MHz | 400MHz |
| 0 | 1 | 1 | 0 | 8000us | 16384 | 2500MHz | 1000MHz |

| Sampling Number |  |  |  |  |
| --- | --- | --- | --- | --- |
| code |  |  |  | description |
| 0 | 0 | 0 | 0 | Base sampling number |
| 0 | 0 | 0 | 1 | 1/2 sampling number |
| 0 | 0 | 1 | 0 | 1/4 sampling number |
| 0 | 0 | 1 | 1 | 1/8 sampling number |

| Tx Channels |  |  |  |  |
| --- | --- | --- | --- | --- |
| code |  |  |  | description |
| 0 | 0 | 0 | 0 | spare |
| 0 | 0 | 0 | 1 | First channel only |
| 0 | 0 | 1 | 0 | Second channel only |
| 0 | 0 | 1 | 1 | Both Channels |

|  | Rx Channels |  |  |  |
| --- | --- | --- | --- | --- |
| Channels | values | One Channel | Two Channels | Four Channels |
| Rx channel 1 | 1: active, 0: inactive |  |  |  |
| Rx channel 2 | 1: active, 0: inactive |  |  |  |
| Rx channel 3 | 1: active, 0: inactive |  |  |  |
| Rx channel 4 | 1: active, 0: inactive |  |  |  |

Specifications for different Rx channels

Single Rx Channel

| Sweep time | Base clock frequency | Single step | Base Sampling Number | Maximum sweepover | Writing cycle Sweepcounter |
| --- | --- | --- | --- | --- | --- |
| 125us | 8.192MHz | 122ns | 256 | 1024 | 2048 steps |
| 250us | 8.192MHz | 122ns | 512 | 2048 | 4096 steps |
| 500us | 8.192MHz | 122ns | 1024 | 4096 | 8192 steps |
| 1000us | 8.192MHz | 122ns | 2048 | 8192 | 16384 steps |
| 2000us | 8.192MHz | 122ns | 4096 | 16384 | 32768 steps |
| 4000us | 8.192MHz | 122ns | 8192 | 32768 | 65536 steps |
| 8000us | 8.192MHz | 122ns | 16384 | 65536 | 131072 steps |

Dual Rx Channels

| Sweep time | Base clock frequency | Single step | Base Sampling Number | Maximum sweepover | Writing cycle |
| --- | --- | --- | --- | --- | --- |
| 125us | 8.192MHz | 122ns | 128 | 1024 | 2048 steps |
| 250us | 8.192MHz | 122ns | 256 | 2048 | 4096 steps |
| 500us | 8.192MHz | 122ns | 512 | 4096 | 8192 steps |
| 1000us | 8.192MHz | 122ns | 1024 | 8192 | 16384 steps |
| 2000us | 8.192MHz | 122ns | 2048 | 16384 | 32768 steps |
| 4000us | 8.192MHz | 122ns | 4096 | 32768 | 65536 steps |
| 8000us | 8.192MHz | 122ns | 8192 | 65536 | 131072 steps |

Quad Rx Channels

| Sweep time | Base clock frequency | Single step | Base Sampling Number | Maximum sweepover | Writing cycle |
| --- | --- | --- | --- | --- | --- |
| 125us | 8.192MHz | 122ns | 64 | 1024 | 2048 steps |
| 250us | 8.192MHz | 122ns | 128 | 2048 | 4096 steps |
| 500us | 8.192MHz | 122ns | 256 | 4096 | 8192 steps |
| 1000us | 8.192MHz | 122ns | 512 | 8192 | 16384 steps |
| 2000us | 8.192MHz | 122ns | 1024 | 16384 | 32768 steps |
| 4000us | 8.192MHz | 122ns | 2048 | 32768 | 65536 steps |
| 8000us | 8.192MHz | 122ns | 4096 | 65536 | 131072 steps |

Following are copies from PUP_EN24C_T2R4_Manual.